The vehicle has met with an accident three days after its purchase. Following damages were noted:-

- Body Shell:- Front both side aprons bent/twisted, the engine has shifted back wardly due to which dash panel is pushed back side & lower floor was got bent. Right rear Qtr. Panel/body panel dented. Both side doors not functioning. Body shell de-aligned for which reason new body shell is recommended.

- Front W/S Glass broken.

- Front Bumper, head lights, grills, fog lights, indicator lights broken.

- Radiator/Condenser assy. pressed damaged.

- Fan & Shroud assy. bent.

- Steering wheel bent.

- Dash board cracked.

- L/S Co-Driver passenger seat bent.

- Rear Seats pressed.

- Air Bag, steering & Co-Passenger air bag actuated/opened, seat belts modules actuated.

- Driver Seat & its Co-Passenger seat bent/de-aligned.

The Front portion of the vehicle was raised on the lift and following damages were further noted.

- Both side suspension upper/lower bent.

- Both side shockers bent.

- Support rod bent.

- Silencer pipe pressed/bent.

- Brake booster pressed.

- Wiring cut at places.

- Steering rack assy. bent.

- Gear box housing crack/broken.

- ABS damaged – pipes bent/pressed.

- AC & Brake Pipes broken.

- Engine & Gear box assy. have shifted backwardly significantly.

The above are the externally visible damages and there is every possibility that some other damages may came to light when the vehicle is dismantled for repairs and which shall increase the liability of the underwriters.