EXTERNALLY VISIBLE DAMAGES:

- Front W/S glass broken; Wiper blade & arms bent.

- Roof Glass broken.

- Front glass both pillar bent.

- Sheet metal portion of roof dented.

- Bonnet centre dented & deshaped.

- Dash board & instrumental panel scratched.

- Both front air bags actuated – modules actuated.

- R/S Front door dented.

- R/S Rear view mirror broken.

- R/S Front door glass broken; Weatherstrips bent.

- R/S Front Fender dented.

- Front Deflector panel – plastic broken.

- Front Bumper broken. Fog lights broken.

- Other plastic linings broken.

- Both head lights broken – fastening clips broken.

- Left side front fender dented.

- Rear View mirror L/S broken.

- Front door dented & handle broken on Left side.

- Rear Door dented – L/S

- Rear Qtr. Panel dented – L/S

- Rear Bumper clips broken.

- Rubber/Plastic mouldings of door damaged.

- Door Plastic strips bent.

- Inner parts viz. glass winding machines bent.

- L/S Front door glass broken.

- Front Grill broken.

- Front R/S Wheel Alloy scratched/bent.

- Right side Front Apron assy. bent at front edge

- R/S Front Suspension Assy. needs check.

- Front Tie member bent at right side joint with apron assy.

- Roof Lining broken.

- Roof Glass broken and their side frames also bent/damaged.

- Radiator/Condenser assy. needs check.

- Right side front tyre brusted – The vehicle hit the foot path due to which R/S suspension and other parts seems effected – needs check.

The Front side of the vehicle was lifted with the crane inorder to check the under-chassis assys. Following main parts were found bent/broken.

- Steering Rack assy. broken.

- Right side long member of chassis frame extensively bent.

- Suspension arms bent.

- Tapet Cover(Plastic broken).

- Sensor damaged. Plastic/rubber pipe broken.

- Right side Apron assy. bent/twisted.