FOLLOWING DAMAGES ARE NOTED:-

- Cabin:- Cabin of the vehicle is totally damaged, the wooden pcs. are scattered on the hill surface.

- Cowl Assy:- Cowl assy. of the vehicle is badly damaged. Front W/S glass frames, lower supporting members; Floor side bonnet assy; Bonnet centre; Dash board etc are damaged in such a way that the cowl assy. is not economically/satisfactory repairable.

- Chassis Frame:- Chassis Frame bent at multiple points. Engine founds; Gear box foundations broken.

- Main Tank:- All sides of the main tank dented & deshaped, side de-canting pipes dented/bent. Upper platform bent/twisted. U. Bolts broken; rubber pads broken.

- Pipes:- All diesel, brake, Air pipes broken/bent/twisted.

- Battery:- Battery broken – battery cables broken.

- Electrical Wiring:- Complete wiring broken.

- Brake:- Brake booster, air tanks broken/dented.

- Front Suspension:- Front Spring leaves assy. intact with chassis frame; tie rods; drag links; tie rod ends broken, steering arms bent; front axle, tie rod de-aligned

- Rear Suspension:- Rear Suspension (Rear Differential tube) may be de-aligned; U/C Bolts broken; Spring leaves assy. broken.

- Propeller Shaft:- Both front/rear propeller shafts bent; c-bearing broken; male-female unit of rear propeller shaft broken.

- Engine Assy:- Oil Sump (Chamber) dented; water body broken; water pipes dented; alternator broken, self starter broken. Both the items are fixed with the engine assy. clutch assy. broken main shaft of gear box broken. However gear box externally ok.

- Radiator assy:- Radiator assy. badly damaged. Fan broken.

- Steering:- Steering wheel broken. Steering column U/J cross, steering rod bent/de-aligned. Steering box OK.