DAMAGES:-

1.	CABIN:- Driver Cabin of the vehicle totally broken. Both side door, roof, back side totally damaged.

2.	Cowl Assy.:- Cowl assy. damaged. Both front cowl sheets, head lights; floor, dash panel; Lower supporting angles of the cowl assy. bent.

3.	Main Tank:- Main tank dented/deshaped side decanting pipes dented/pressed, lever bent. Upper platform dented. Pipes bent.

4.	Suspension assy. Front suspension main spring leaves broken. U/C bolts bent; hanger/shackle assy. broken. Front axle de-aligned, tie rod & link rods bent, tie rod ends broken. Shocker bent.

5.	Rear Suspension: Rear Suspension Spring leaves broken, U/C Bolts/clamps broken/bent. Differential tube seems de-aligned. No. not readable. Brake Pipes broken. Metallic pipes broken/pressed.

6.	Electrical System:- Complete wiring of cowl assy. damaged. Battery cables broken. Battery broken; lights broken. Indicator & other electric system damaged. Wiper broken. Switches broken.

7.	Steering System:- Steering wheel bent, steering rod bent. U-Cross broken, steering box seems intact, its No. could not be noted not visible.

8.	Chassis Frame:- Chassis frame bent/de-aligned, its cross-member bent. The chassis number is photographed but not clearly visible.

9.	Cooling System:- Radiator assy. damaged coolant drain out. Fan broken water body pulley bent. Fan belt broken.

10.	Engine assy. Oil Sump dented, all the diesel pipes bent/fuel pump damaged. Needs checking thoroughly  clutch housing broken.

11.	Gear Box: Gear Box seems OK.

12.	Wheel Rims/Tyres:- Wheel Rim seems de-aligned, Tyres No. were checked and are 50% used condition.