1.	Front Bumper/grill assy. broken.

2.	Both head lights broken. Indicator & Fog Lights broken.

3.	Bonnet Center & left side fender badly damaged.

4.	Upper member broken. R/S Fender dented.

5.	Fender Lining both sides broken.

6.	Front W/S glass cracked.

7.	Left side Front door dented.

8.	Left side rear door & Rear Qtr. Panel slightly dented. Running board dented.

9.	Right side apron assy. & L/S apron assy. bent/sheet twisted.

10.	Right side Rear Qtr. Panel & body panel dented.

11.	Rear View mirror both side broken.

12.	Radiator & Condenser assy. bent/pressed inwardly. Radiator shroud fan broken.

13.	Instrumental panel broken (cracked, its consoles displaced from their positions & uprooted.

14.	The under-chassis portion of the vehicle was also reported damaged and as such the vehicle was moved on to lift. Raised on lift and following other damages were noted.

15.	A.C. Pipes pressed/damaged.

16.	Gear Box Housing broken.

17.	Gear Foundations & Engine Foundations broken/bent

18.	Wheel Rims front both bent.

19.	Shocker assy. seen bent.

20.	Drive axles both sides de-aligned.

21.	Oil Sump dented.

22.	Rear Axle got hit.

23.	Wiper arms/blade bent/twisted.

24.	Gear Box housing broken.

25.	Power Steering rack assy. found leaking. Crack in the body could not visible at the time of inspection, but it is assured that it might be effected due to accident.

26.	Front Suspension assy. damaged.