DAMAGES:-

1.	The Left front side of the vehicle has hit the parapet due to which L/S front suspension, Fender are badly damaged. The front L/S drive axle has came out and the left side wheel is hanging. Front wheel rim bent & tyre torn.

2.	Fender dented; Apron L/S bent.

3.	Front Bumper broken; Head Light broken.

4.	Front grill broken.

5.	Radiator & Condenser assy. pressed inwardly.

6.	Front tie member bent.

7.	Bonnet assy. dented & deshaped.

8.	R/S Head Light inner fastening clips broken.

9.	R/S Rear View mirror broken.

10.	W/S Glass broken/Cracked.

11.	Right side body panel dented above the door frame.

12.	Roof dented & deshaped at the front side.

13.	Right side fender dented.

14.	Both front air bags got opened (visible in the photos)

15.	R/S Drive axle came out.

16.	Gear Box assy. housing broken. The impact of accident has also broken the tooth of the gears (visible in the photos). The Gear box of the vehicle is badly damaged and needs replacement.

17.	Suspension arms of L/S suspension bent.

18.	Radiator shroud fallen down.