EXTERNALLY VISIBLE DAMAGES:

1.	Cabin & Load Body:- Totally damaged.

2.	Cowl Assy.:- Front face cowl panels dented; cowl foundations broken; grill dented; floor dented/twisted/de-aligned; Dash panel bent/twisted meter panel & meters broken – not repairable.

3.	Chassis Assy.:- Chassis Assy. of the vehicle is extensively bent – needs replacement.

4.	Alternator Assy.:- Alternator assy. broken, central shaft bent, fan pulley bent.

5.	Engine assy:- Tapet cover broken; Fan belt broken; Alternator assy.. broken & Central Shaft bent; lower oil sump dented. It seems that when the vehicle fell down and got held up, the driver of the vehicle was injuried, the oil might have leaked out.

6.	Cooling System:- Radiator frame along with radiator bent – Coolant tanks seems OK.

7.	Inter-cooler:- Inter-cooler assy. which is fixed with the radiator also bent.

8.	Battery:- Exide battery cracked.

9.	Electrical System:- Whole of wiring front; dash panel & rear all broken/damaged Fuse box broken.

10.	Steering System:- Steering wheel bent; steering column uprooted from the steering box. Colum bent; U/C bent.

11.	Front Suspension:- The repairer has clubbed all the parts of the front suspension assy. under one head, whereas on inspecting the vehicle it was observed that main tie rod is bent; tie rod ends broken; steering arms bent; shocker assy. bent; Front axle de-aligned. However the exact amount of the damaged parts can only be ascertained when the vehicle is subjected to repairs. However I am taking in consideration the tentative amount towards the replacement of apparently damaged parts and its repair charges.

12.	Rear Suspension:- From the photos attached it is clearly visible that the spring leaves are dislocated. The spring leaves assy. needs opening, replacing the broken/crack leaves, re-cambering & re-assy.

13.	Propeller Shaft:- Propeller shaft seems bent.

14.	Diesel Tank:- Diesel tank along with its brackets badly dented/bent/broken. Diesel pipes dented.

15.	Air Tank:- Air tank dented.

16.	Brake System:- Brake system totally damaged.

17.	Air System:- Air System Damaged.

18.	All the pipes (brake – Air) broken, both input & output air pipes broken. Turbo assy. seems damaged.

19.	Gear Box; Clutches & Pressure plate assy. seems OK. Gear box lever bent.